## 📘 Master Guide: OAI 5G Core (v2.2.1) Deployment on GCP

### Phase 1: Safe Docker Installation (GCP Specific)

Google Cloud VMs typically use the 10.x.x.x IP range. Installing Docker out-of-the-box can create routing conflicts that instantly lock you out of SSH. We must force Docker to use the 192.168.x.x subnet before installation.

1. Create the Safe Docker Configuration

sudo mkdir -p /etc/docker

sudo nano /etc/docker/daemon.json

Paste this exact configuration:

JSON

{

"bip": "192.168.100.1/24",

"default-address-pools": [

{

"base": "192.168.101.0/16",

"size": 24

}

]

}

(Save and exit).

2. Install Docker and Compose Plugin

curl -fsSL https://get.docker.com -o get-docker.sh

sudo sh get-docker.sh

sudo apt-get install docker-compose-plugin -y

3. Add User to Docker Group (This allows running docker without sudo. You must log out and log back in for it to take effect).

sudo usermod -aG docker $USER

### Phase 2: Clone & Pull Stable Release

Avoid the develop branch. We strictly want the v2.2.1 stable release. Note that as of v2.2.0, OAI deprecated the VPP version of the UPF in favor of their modernized standard UPF.

1. Clone the Specific Tag

git clone -b v2.2.1 https://gitlab.eurecom.fr/oai/cn5g/oai-cn5g-fed.git

cd oai-cn5g-fed/docker-compose

2. Pull the Pre-Built v2.2.1 Images (Pulling these manually ensures Docker doesn't try to guess or pull missing dependencies later).

docker pull oaisoftwarealliance/oai-amf:v2.2.1

docker pull oaisoftwarealliance/oai-nrf:v2.2.1

docker pull oaisoftwarealliance/oai-smf:v2.2.1

docker pull oaisoftwarealliance/oai-udr:v2.2.1

docker pull oaisoftwarealliance/oai-udm:v2.2.1

docker pull oaisoftwarealliance/oai-ausf:v2.2.1

docker pull oaisoftwarealliance/oai-upf:v2.2.1

docker pull mysql:9.0

sudo docker pull oaisoftwarealliance/oai-amf:v2.2.1

sudo docker pull oaisoftwarealliance/oai-nrf:v2.2.1

sudo docker pull oaisoftwarealliance/oai-smf:v2.2.1

sudo docker pull oaisoftwarealliance/oai-udr:v2.2.1

sudo docker pull oaisoftwarealliance/oai-udm:v2.2.1

sudo docker pull oaisoftwarealliance/oai-ausf:v2.2.1

sudo docker pull oaisoftwarealliance/oai-upf:v2.2.1

sudo docker pull mysql:9.0

### Phase 3: Configure and Deploy

The default YAML files in the v2.2.1 branch are still hardcoded to pull develop images. We must manually overwrite this to guarantee stability.

1. Force the YAML file to use v2.2.1 We use the standard NRF profile (docker-compose-basic-nrf.yaml). Use sed to find and replace all instances of "develop" with "v2.2.1":

sed -i 's/develop/v2.2.1/g' docker-compose-basic-nrf.yaml

2. Deploy the Core Network Launch the containers in detached mode:

docker compose -f docker-compose-basic-nrf.yaml up -d

3. Verify the Deployment Check the status of the network:

docker ps -a

GET the AMF IP

docker inspect oai-amf | grep IPAddress

EDIT Here

nano openairinterface5g/targets/PROJECTS/GENERIC-NR-5GC/CONF/gnb.sa.band78.fr1.106PRB.usrpb210.conf

Success Criteria: * You should see containers running the v2.2.1 tags (e.g., oai-upf:v2.2.1, oai-amf:v2.2.1).

- Within 30–60 seconds, the database will initialize, and all container statuses should read Up X seconds (healthy).

### Phase 4: Troubleshooting / Tear Down

If you ever make a mistake or need to cleanly restart the core network from scratch, always run this command from the docker-compose folder to delete the containers, networks, and orphaned volumes:

docker compose -f docker-compose-basic-nrf.yaml down -v

Prerequisite: The OAI 5G Core Network (v2.2.1) must be running and healthy before starting the RAN elements.

### Phase 1: Clone and Prepare the Environment

We use the 2024.w38 tag to ensure perfect compatibility with the v2.2.1 Core Network.

1. Clone the Repository and Checkout Tag

cd ~

git clone https://gitlab.eurecom.fr/oai/openairinterface5g.git -b 2024.w38

cd openairinterface5g

2. Initialize the FlexRIC Submodule Critical step to ensure the gNB compiles against the exact matching version of the E2 agent.

cd openair2/E2AP/flexric

git submodule init

git submodule update

### Phase 2: Install Dependencies (Debian 12 Hack)

OAI officially only supports Ubuntu/RHEL for the build_oai script. If running on Debian 12, you must temporarily spoof the OS signature.

1. Spoof Ubuntu 22.04 LTS

sudo cp /etc/os-release /etc/os-release.bak

sudo bash -c 'cat > /etc/os-release <<EOF

PRETTY_NAME="Ubuntu 22.04 LTS"

NAME="Ubuntu"

VERSION_ID="22.04"

VERSION="22.04 LTS (Jammy Jellyfish)"

VERSION_CODENAME=jammy

ID=ubuntu

ID_LIKE=debian

EOF'

2. Install Core OAI Dependencies

cd ~/openairinterface5g/cmake_targets

source oaienv

./build_oai -I

3. Restore the True Debian OS Signature

sudo mv /etc/os-release.bak /etc/os-release

### Phase 3: Build the FlexRIC Service Models

Before compiling the RAN, you must install the SWIG wrappers and build the FlexRIC plugins, or the gNB will crash on startup when trying to load the E2 agent.

1. Install SWIG and Python Dev Tools

sudo apt-get update

sudo apt-get install -y build-essential libpcre2-dev autoconf automake libtool bison flex

sudo apt update

sudo apt install -y python3-dev python3-pip

1.1 Install Swig

cd ~

wget https://prdownloads.sourceforge.net/swig/swig-4.1.1.tar.gz

tar -xvf swig-4.1.1.tar.gz

cd swig-4.1.1

./configure --prefix=/usr

make -j$(nproc)

sudo make install

1.2 Install GCC-12 for Fixing compiling issues

sudo apt update

sudo apt install -y gcc-12 g++-12

sudo update-alternatives --install /usr/bin/gcc gcc /usr/bin/gcc-12 120

sudo update-alternatives --install /usr/bin/g++ g++ /usr/bin/g++-12 120

sudo update-alternatives --config gcc

sudo update-alternatives --config g++

gcc --version

g++ --version

2. Build and Install the Plugins

cd ~/openairinterface5g/openair2/E2AP/flexric/build

# Clear the old, failed CMake cache

rm -rf *

cmake ..

make -j$(nproc)

sudo make install

sudo ldconfig

### Phase 4: Compile the RAN (gNB & UE)

We build for the RF Simulator with the E2 Agent enabled, utilizing Ninja for faster compilation.

cd ~/openairinterface5g/cmake_targets

./build_oai -w SIMU --gNB --nrUE --build-e2 --ninja

### Phase 5: Launch the Network

1. Start the gNB (Terminal 1) We must append --gNBs.[0].min_rxtxtime 4 to relax the scheduling timeline (K2 slot offset). Without this, the software UE will crash during the RACH procedure because it cannot process PUSCH commands fast enough.

cd ~/openairinterface5g/cmake_targets/ran_build/build

sudo ./nr-softmodem -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/gnb.sa.band78.fr1.106PRB.usrpb210.conf --rfsim --sa --gNBs.[0].min_rxtxtime 4 --gNBs.[0].enable_sdap 1

2. Start the UE (Terminal 2) Wait for the gNB to fully boot. The -C flag must be defined in Hz, not kHz.

cd ~/openairinterface5g/cmake_targets/ran_build/build

Run UE 1

sudo ./nr-uesoftmodem -r 106 --numerology 1 --band 78 -C 3619200000 --rfsim --sa -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/ue.conf

sudo ip link set dev oaitun_ue1 down

sudo ip link set dev oaitun_ue1 name oaitun_ue_1

sudo ip link set dev oaitun_ue_1 up

Run UE 2

sudo ./nr-uesoftmodem -r 106 --numerology 1 --band 78 -C 3619200000 --rfsim --sa -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/ue2.conf

sudo ip link set dev oaitun_ue1 down

sudo ip link set dev oaitun_ue1 name oaitun_ue_2

sudo ip link set dev oaitun_ue_2 up

Run UE 3

sudo ./nr-uesoftmodem -r 106 --numerology 1 --band 78 -C 3619200000 --rfsim --sa -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/ue3.conf

sudo ip link set dev oaitun_ue1 down

sudo ip link set dev oaitun_ue1 name oaitun_ue_3

sudo ip link set dev oaitun_ue_3 up

Run UE 4

sudo ./nr-uesoftmodem -r 106 --numerology 1 --band 78 -C 3619200000 --rfsim --sa -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/ue4.conf

sudo ip link set dev oaitun_ue1 down

sudo ip link set dev oaitun_ue1 name oaitun_ue_4

sudo ip link set dev oaitun_ue_4 up

Run UE 5

sudo ./nr-uesoftmodem -r 106 --numerology 1 --band 78 -C 3619200000 --rfsim --sa -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/ue5.conf

Hack to avoid typing rename commands everytime

Inside bash file

#!/bin/bash

if [ -z "$1" ]; then

echo "Usage: ./rename_ue.sh <number>"

exit 1

fi

NEW_NAME="oaitun_ue_$1"

echo "Renaming oaitun_ue1 to $NEW_NAME..."

sudo ip link set dev oaitun_ue1 down

sudo ip link set dev oaitun_ue1 name $NEW_NAME

sudo ip link set dev $NEW_NAME up

echo "Done! You can now launch the next UE."

Make it executable:

chmod +x rename_ue.sh.

Now, your workflow is incredibly fast:

- Start UE 1 $\rightarrow$ Wait $\rightarrow$ Run ./rename_ue.sh 1

- Start UE 2 $\rightarrow$ Wait $\rightarrow$ Run ./rename_ue.sh 2

- Start UE 3 $\rightarrow$ Wait $\rightarrow$ Run ./rename_ue.sh 3

- ...and so on!

3. Verify Connectivity (Terminal 3) Watch the UE logs for [NAS] Setting up interface oaitun_ue1. Once that appears, verify the tunnel and ping the internet to confirm full End-to-End data flow:

ip a | grep oaitun_ue1

ping -I oaitun_ue1 8.8.8.8

To get your network fully synchronized before firing up the FlexRIC server and your LAPF xApp, we had to edit two critical configuration files.

These edits fixed the IP routing and perfectly aligned your Radio Access Network (gNB/UE) with the exact identity parameters expected by your 5G Core Network (the AMF and SMF "bouncers").

Here are the exact files we modified and what we changed in them:

### 1. The gNB Configuration File

File Location:

nano openairinterface5g/targets/PROJECTS/GENERIC-NR-5GC/CONF/gnb.sa.band78.fr1.106PRB.usrpb210.conf

We made three major sets of changes to this file to get the radio tower talking to the AMF and to ensure your LAPF scheduler could read the QoS priority levels:

- Network Slice & Identity (PLMN): We updated both the Global gNB ID block (top of the file) and the Cell-Level block (middle of the file) to perfectly match the Core Network's database:

  - mcc = 208

  - mnc = 95

  - tracking_area_code = 0xa000 (or 40960)

  - snssaiList = ({ sst = 222; sd = 0x7b; })

- IP Routing: We pointed the gNB to your Docker bridge network so it could physically reach the Core.

docker inspect -f '{{range .NetworkSettings.Networks}}{{.IPAddress}}{{end}}' oai-amf

  - amf_ip_address = ({ ipv4 = "192.168.70.133"; })

  - GNB_IPV4_ADDRESS_FOR_NG_AMF = "192.168.70.129/26"

  - GNB_IPV4_ADDRESS_FOR_NGU = "192.168.70.129/26"

- The SDAP Fix (Crucial for LAPF): We added a flag at the very top inside the gNBs block to force the gNB to create the QoS routing tables. Without this, your Python script threw the Entity not found error.

  - enable_sdap = "yes"; (Or forced via the --gNBs.[0].enable_sdap 1 command-line flag).

### 2. The UE Configuration File

File Location: ~/openairinterface5g/targets/PROJECTS/GENERIC-NR-5GC/CONF/ue.conf

We completely replaced the uicc0 (SIM card) block in this file. The AMF was initially rejecting the UE for having an unauthorized SIM, and the SMF was refusing to hand out an IP address because the UE was asking for the wrong data network.

We injected these exact credentials:

uicc0 = {

imsi = "208950000000031";

key = "0C0A34601D4F07677303652C0462535B";

opc= "63bfa50ee6523365ff14c1f45f88737d";

dnn= "default";          // <--- This specific word is what finally got you an IP address!

nssai_sst= 222;

nssai_sd= 123;         // (123 is the decimal equivalent of 0x7b)

}

Once those two files are saved with these exact parameters, your OAI infrastructure is permanently synced. You can safely launch the Core, the gNB, and the UE, and they will establish the oaitun_ue1 IP tunnel automatically every single time!

## 📘 Master Guide Part 2: O-RAN FlexRIC & Python xApp Environment

Prerequisite: OAI 5G Core (v2.2.1) and OAI RAN (2024.w38) must be compiled and ready.

### Phase 1: The "Source Code Lobotomy" (Database Fix)

By default, the FlexRIC C backend forces all telemetry to be written to a local SQLite database in /tmp/. At 5G speeds, this instantly locks the disk (SQLITE_BUSY) and crashes the xApp. We must manually disable the tripwire.

1. Edit the SQLite Wrapper

nano ~/openairinterface5g/openair2/E2AP/flexric/src/xApp/db/sqlite3/sqlite3_wrapper.c

2. Silence the Assertion Search for the insert_db function (around line 320). Find this line and comment it out using //:

// assert(rc == SQLITE_OK && "Error while inserting into the DB. Check the err_msg string for further info");

(Save and exit).

### Phase 2: Compile and Install FlexRIC

With the crash loop disabled, we compile the Near-RT RIC and the Python (SWIG) SDK.

1. Build the RIC and Plugins

cd ~/openairinterface5g/openair2/E2AP/flexric

mkdir -p build && cd build

cmake ..

make -j$(nproc)

sudo make install

sudo ldconfig

2. Link the Python Environment You must tell Ubuntu where to find the generated Python libraries. (Note: You will need to run this command in your xApp terminal every time you log in, or add it to your ~/.bashrc).

export PYTHONPATH=/usr/local/lib/flexric:$PYTHONPATH

Step 1: Start the Near-RT RIC Server

The RIC is the central brain. It sits between your gNB and your xApps. It receives metrics from the gNB and forwards your xApp's scheduling decisions back to the gNB.

Open a new terminal (Terminal 4).

Navigate to the compiled RIC directory:

cd ~/openairinterface5g/openair2/E2AP/flexric/build/examples/ric

Start the RIC server:

./nearRT-RIC

What to expect: It will boot up and bind to port 36421 (the standard E2 port). It is now actively listening for base stations.

### Phase 3: The Clean LAPF Monitor Script

To prevent overwhelming the E2 interface during development, we use a custom Python script that strictly subscribes to the MAC Service Model at a safe interval.

1. Create the Script

cd ~/openairinterface5g/openair2/E2AP/flexric/build/examples/xApp/python3

nano lapf_monitor.py

2. Paste the Foundation Boilerplate

import xapp_sdk as ric

import time

class MACCallback(ric.mac_cb):

def __init__(self):

ric.mac_cb.__init__(self)

def handle(self, ind):

if len(ind.ue_stats) > 0:

t_now = time.time_ns() / 1000.0

t_diff = t_now - (ind.tstamp / 1.0)

print(f"MAC Ind tstamp = {ind.tstamp} | latency = {t_diff:.2f} μs | RNTI = {ind.ue_stats[0].rnti}")

ric.init()

conn = ric.conn_e2_nodes()

assert(len(conn) > 0), "Error: No E2 nodes connected!"

mac_hndlr = []

for i in range(0, len(conn)):

mac_cb = MACCallback()

# Interval set to 10ms to ensure stability

hndlr = ric.report_mac_sm(conn[i].id, ric.Interval_ms_10, mac_cb)

mac_hndlr.append(hndlr)

time.sleep(1)

try:

while ric.try_stop == 0:

time.sleep(1)

except KeyboardInterrupt:

pass

for i in range(0, len(mac_hndlr)):

ric.rm_report_mac_sm(mac_hndlr[i])